RUSLE

Before implementing the RUSLE model ensure that all input datasets:

- Are in the same coordinate system — WGS 84 (EPSG:4326).

- Are resampled to a uniform spatial resolution of 90 m (to match the DEM).

- Are clipped to the study area (AOI) for computational efficiency.

- Link to gee output for 1 year: https://code.earthengine.google.com/1386566e9921d6da61aeaa80e7ce2aa2?noload=true

RUSLE Parameters and Data Sources

| Parameter | Name | Represents | Data Source / Input (Google Earth Engine) | Spatial Resolution | Temporal Coverage | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| R | Rainfall–Runoff Erosivity Factor | Energy & intensity of rainfall causing soil detachment | CHIRPS Daily Precipitation (UCSB/CHG) | 0.05° (~5 km) | 2014–Present | Used for deriving the annual R factor using rainfall erosivity relation. |
| K | Soil Erodibility Factor | Soil’s inherent vulnerability to erosion based on texture and structure | OpenLandMap: Soil Texture Class (USDA-TT) | 250 m | 2014–2018 | Use 2018 data for all subsequent years. |
| LS | Topographic Factor (Slope Length & Steepness) | Influence of slope and flow accumulation on erosion | CGIAR SRTM90_V4 (DEM) | 90 m | 2000 | Used consistently throughout due to low temporal variability. |
| C | Cover Management Factor | Protective effect of vegetation and land cover | Google Dynamic World v1 (Sentinel-2) | 10 m | 2014–Present | NDVI-based C factor derived from Sentinel-2 (B8, B4). |
| P | Support Practice Factor | Impact of land management and conservation practices | Derived empirically from Dynamic World land cover + slope data | Unitless | — | Based on slope-dependent values for cropland and other land covers (see below). |

R Factor – Rainfall Erosivity (CHIRPS)

Each rainfall event carries kinetic energy, which depends on:

  - Rainfall amount (mm)

  - Rainfall intensity (mm/h)

  - Drop size and velocity

- High-intensity rainfall produces large, fast-falling drops, which have greater momentum and cause more soil particle detachment when they hit bare soil.

- this simplified empirical relation is applied:

where:

  - RRR = rainfall erosivity factor (MJ·mm·ha⁻¹·h⁻¹·yr⁻¹)

  - PPP = annual precipitation (mm)

- This equation was derived from statistical correlations between measured erosivity and total annual rainfall for Indian catchments (often from sources like Wischmeier & Smith 1978, or Indian empirical calibrations).

- It means that:

  - When rainfall increases, erosivity increases almost linearly.

  - Even with low rainfall, there’s a base erosivity (~79) because occasional intense storms still cause erosion.

K Factor – Soil Erodibility (OpenLandMap)

- Soil erodibility is a function of intrinsic soil properties, mainly:

  - Texture — proportion of sand, silt, clay

  - Organic matter content

  - Soil structure (granular, blocky, etc.)

  - Permeability or infiltration capacity

- Each of these controls how strongly soil particles bind together and how resistant they are to detachment.

  - Silty soils → very erodible (high K)

  - Clayey soils → cohesive, less erodible (low K)

  - Sandy soils → infiltration high, but poor cohesion (moderate K)

- The dataset (e.g., OpenLandMap) classifies each pixel’s soil texture numerically (e.g., 1 = clay, 2 = sandy clay loam, … 12 = sand).

- The code uses a conditional expression (a lookup table):

  - If the soil code is greater than 11 → K=0.0053K = 0.0053K=0.0053

  - If the soil code is greater than 10 → K=0.0170K = 0.0170K=0.0170

  - If the soil code is greater than 9 → K=0.045K = 0.045K=0.045

  - … and so on, until each texture class has a representative K-value.

These K-values come from empirical literature, representing average erodibility for that soil type.

- The final map (KFactor Map) helps you visualize spatial variation in soil erodibility:

  - It highlights fragile soils prone to erosion.

  - It helps correlate soil characteristics with terrain or land cover patterns.

  - It’s an input layer for multiplying in the RUSLE model.

LS Factor – Topographic Factor (Slope Length & Steepness) (SRTM)

- the DEM (Digital Elevation Model) provides elevation values for each pixel.

- from this, the local slope angle (in degrees) is derived — how steep the land surface is.

- RUSLE formulations use slope in percent (%). The slope % is used in LS Computation

- implified empirical equations estimate LS just from slope (%) and an assumed average slope length (like 500 m).

where:

  - S = slope in %

  - L = constant slope length (assumed 500 m for your basin)

| Code term | Formula meaning | What it represents |
| --- | --- | --- |
| LS4 = sqrt(500/100) |  | Slope length effect → how long the slope is |
| LS3 = slope × 0.53 | 0.53S | Steepness → how steep it is |
| LS2 = slope × slope × 0.076 | 0.076S | Exaggerates steep areas (steep slopes increase erosion rapidly) |
| LS1 = LS3 + LS2 + 0.76 |  | Combines slope effects into one value |
| LS = LS1 × LS4 | Multiply both effects | Final LS map |

LS = (effect of steepness) × (effect of slope length)

C Factor – Cover and Management Factor (NDVI)

- the C factor represents the effect of vegetation, crop cover, and land management practices on soil erosion.

- It quantifies how much soil is protected or exposed to rainfall.

- We use NDVI (Normalized Difference Vegetation Index) from Sentinel-2 images.

- B8 (NIR): measures how much vegetation reflects near-infrared light.

- B4 (Red): measures how much red light plants absorb.

- converts NDVI into C using an exponential formula:

- This means:

  - When NDVI is high (lots of vegetation), CCC becomes small → low erosion.

  - When NDVI is low (bare land), CCC becomes large → high erosion.

Then the code normalizes C between 0 and 1 (so all pixels are on the same scale).

- Displays a C-factor map, showing where land is protected or bare.

P Factor (Support Practice Factor)

- The P Factor (Support Practice Factor) represents how land management and conservation practices help reduce soil erosion.

- P is a ratio that compares soil loss with a support practice to soil loss without it:

So:

- P=1P = 1P=1 → no conservation → maximum erosion

- P<1P < 1P<1 → some conservation → reduced erosion

- Dynamic World v1 gives land cover classes from Sentinel-2 at 10 m:

  - Crops

  - Trees

  - Grass

  - Shrubs

  - Built-up

  - Bare land

  - Water

- if we base it on Dynamic World land cover and slope (%), we can express it like this:

- This is cause:

  - Croplands on flatter slopes (where contour bunds or terraces are possible) → lower P → less erosion.

  - Croplands on steeper slopes (where conservation is difficult) → higher P → more erosion.

  - Forest, built-up, bare, or water → P = 1.0 because there are no artificial conservation measures (forests are naturally protected; the rest are unmanaged).

Classifying erosion severity: RUSLE

| Class | Soil Loss (t/ha/yr) | Severity | Typical Meaning |
| --- | --- | --- | --- |
| 1 | < 5 | Slight | Minimal erosion, stable land |
| 2 | 5–10 | Moderate | Manageable erosion, early degradation |
| 3 | 10–20 | High | Noticeable soil removal, needs management |
| 4 | 20–40 | Very High | Active rills, gully formation possible |
| 5 | > 40 | Severe | Major erosion, serious land degradation |

## Model Outputs and Interpretation

After deriving all the RUSLE input parameters (R, K, LS, C, and P), the final soil erosion potential was computed as:

where A represents the annual soil loss in tonnes per hectare per year (t·ha⁻¹·yr⁻¹).
The outputs generated from this computation include both spatial maps and statistical summaries for interpretation and reporting.

### 1. Soil Loss Map

This is the core RUSLE output, representing the estimated annual soil loss for each pixel (in t·ha⁻¹·yr⁻¹).

Details:

- Layer Name: Soil Loss

- Units: Tonnes per hectare per year (t·ha⁻¹·yr⁻¹)

- Visualization Palette: Blue → Red gradient
(Blue = low erosion; Red = high erosion)

Interpretation:

- Low values (blue–green): Stable or vegetated areas with minimal soil loss.

- High values (orange–red): Steep, bare, or poorly managed zones prone to erosion.

This map highlights spatial heterogeneity in erosion risk and forms the basis for prioritizing soil conservation efforts.

### 2. Classified Erosion Severity Map

To simplify interpretation, the continuous soil loss values were categorized into five severity classes following standard classification schemes.

| Class | Soil Loss (t/ha/yr) | Category | Description |
| --- | --- | --- | --- |
| 1 | < 5 | Slight | Minimal erosion, stable land |
| 2 | 5–10 | Moderate | Manageable erosion, minor degradation |
| 3 | 10–20 | High | Active sheet erosion |
| 4 | 20–40 | Very High | Rills and visible soil removal |
| 5 | > 40 | Severe | Serious erosion and gully formation |

Details:

- Layer Name: Soil Loss Class

- Palette: 🟦 Slight → 🟥 Severe (same color ramp as soil loss map)

- Purpose: Simplifies reporting, decision-making, and map-based communication.

### 3. Basin and Sub-Basin Statistics

To quantify erosion patterns at the watershed level, spatial statistics were computed:

Metrics:

- Mean annual soil loss (t·ha⁻¹·yr⁻¹) for the entire basin (AOI).

- Area under each erosion class (km²), representing the spatial extent of severity zones.

Methodology:

- Multiply each classified erosion raster by pixel area to calculate the area of each class.

- Aggregate the pixel areas to determine total area per class.

- Convert pixel counts to square kilometers (km²) for reporting.

This allows for sub-basin prioritization and erosion hotspot identification.

### 4. Pie Chart of Erosion Severity

A pie chart was generated to visually represent the percentage area of each erosion class within the basin.
It provides an intuitive overview of erosion distribution and dominance of specific severity levels.

Example Interpretation:

| Class | Area (km²) | Percentage (%) |
| --- | --- | --- |
| Slight | 45 | 40% |
| Moderate | 35 | 30% |
| High | 20 | 20% |
| Very High | 10 | 9% |
| Severe | 2 | 1% |

“Approximately 70% of the basin experiences slight to moderate erosion, whereas about 10% faces very high to severe soil loss, indicating zones requiring urgent management interventions.”

### 5. Map Legend and Visualization Scheme

A consistent color legend is used for visual clarity across all RUSLE outputs.

| Color | Severity | Soil Loss (t/ha/yr) |
| --- | --- | --- |
| 🟦 Blue | Slight | < 5 |
| 🟩 Green | Moderate | 5–10 |
| 🟨 Yellow | High | 10–20 |
| 🟧 Orange | Very High | 20–40 |
| 🟥 Red | Severe | > 40 |

Legend Use:
This standardized palette ensures that erosion severity is easily identifiable across spatial and temporal analyses.