Spatio-temporal Soil Erosion Assessment in the Mula-Mutha Catchment, Pune

Using RUSLE and Python

Synopsis By: Bhavya Singh

PRN No: 2415120013

M.Sc. Geoinformatics

Under the Guidance of

Dr. Lakshmi Kantakumar N.

Bharati Vidyapeeth (Deemed to be University) Institute of Environment Education and Research (BVIEER) Pune,

Maharashtra, India 2023-2025

Table of Contents

1.Introduction	3

2.Literature Review	3

3.Research Gap	3

4. Aim and Objectives	4

5. Study Area Description	4

6. Datasets and Methodology	4

7. Probable Outcomes	5

8. Work Plan	5

9. References	5

# Introduction

Soil erosion by water is a major environmental concern due to climate change and untimely rains in most part of the country affecting agricultural productivity, water quality, and reservoir siltation. The Revised Universal Soil Loss Equation (RUSLE) offers a widely adopted, cell-based framework to estimate soil loss by combining rainfall erosivity (R), soil erodibility (K), topographic (LS), cover-management (C), and support-practice (P) factors. However, most studies in India use GIS or Google Earth Engine with partial automation and limited temporal scope.

This synopsis outlines a fully Python-driven, decadal (2011–2020) assessment of soil erosion in the Mula–Mutha catchment, Pune, leveraging open-source libraries for reproducibility and scaled analyses.

__________________________________________________________________________________

# Literature Review

Over the past few decades, researchers have steadily refined the Revised Universal Soil Loss Equation (RUSLE) to make soil-erosion assessments both more accurate and more accessible. The foundational guide by Renard et al. [1] laid out the core framework—defining how rainfall erosivity (R), soil erodibility (K), slope factors (LS), cover management (C), and support practices (P) work together. Building on that, Shinde et al. [2] demonstrated in the Western Ghats how coupling RUSLE with GIS can pinpoint erosion-prone areas, making it easier for planners to focus conservation efforts.

In Kerala, Prasannakumar and colleagues [3] showed the value of integrating high-resolution remote sensing imagery, highlighting year-to-year changes in erosion that static surveys often miss. A similar approach in the Nethravathi Basin by Ganasri & Ramesh [4] underscored the importance of up-to-date land-cover data for accurately estimating the C-factor.

More recently, cloud platforms have transformed our workflow. Papaiordanidis & Jirka [5] packaged RUSLE entirely within Google Earth Engine, allowing teams to run multi-year analyses in minutes rather than weeks. Meanwhile, Pandey et al. [6] reminded us that digital elevation model resolution critically shapes our LS-factor outputs—coarser DEMs can mask steep slopes that drive erosion hotspots.

Looking forward, hybrid approaches promise even greater insight. Chakrabortty et al. [7] combined RUSLE with machine-learning classifiers to flag vulnerable zones with improved precision, while Ghosh et al. [8] illustrated a fully Python-based, decadal assessment that marries IMD rainfall records with ESA WorldCover maps to reveal long-term trends.

Together, these studies chart a clear path: by embracing cloud-computing, up-to-date remote sensing, and smart analytics, our Mula–Mutha catchment assessment can deliver robust, actionable insights for watershed managers and policy makers alike.

__________________________________________________________________________________

# Research Gap

Although prior work has automated RUSLE components on cloud platforms or hybrid GIS-ML frameworks, there remains:

- No fully open-source, Python-centered pipeline for decadal soil-loss computation at 30 m resolution.

- Limited application of year-by-year RUSLE analyses in the Mula–Mutha catchment context.

- Absence of reproducible Google Collab notebooks integrating all RUSLE factors from raw data ingestion to change-detection outputs.

__________________________________________________________________________________

4. Aim and Objectives
4.1 Aim:
To develop an automated, Python-based RUSLE workflow for spatio-temporal soil erosion analysis in the Mula–Mutha watershed (2011–2020), using open-source tools and public datasets to quantify decadal changes.

4.2 Objectives:

- To develop a fully automated RUSLE model workflow for a decade analysis

- To assess spatio-temporal variability in soil erosion

__________________________________________________________________________________

5. Study Area Description

The Mula–Mutha catchment (18°17′–18°31′ N, 73°25′–73°53′ E) drains ~600 km² in Pune district, Maharashtra. Elevation ranges from 560 m in downstream plains to 1,200 m in the Western Ghats foothills. Land use includes urban, agricultural, and forested zones, making it ideal for assessing varied erosion dynamics.

__________________________________________________________________________________

6. Datasets and Methodology

6.1 Datasets

- Rainfall (R): Daily gridded IMD data (0.25°) for 2011–2020

- DEM (LS): 30 m SRTM v3 tiles, mosaicked and clipped to catchment

- Soil (K): FAO soil-erodibility map, resampled to 30 m

- Land Cover (C): ESA WorldCover annual products (10 m), reclassified into C values

- Support Practices (P): Assumed P = 1 across the catchment

6.2 Methodology Flowchart

The methodology begins by ingesting all spatial data—DEM, land-use/land-cover, soil and rainfall rasters—and masking them to the Mula–Mutha catchment boundary. Next, rainfall erosivity (the R-factor) is estimated: missing IMD values are interpolated. Topographic effects (the LS-factor) are then derived to compute slope and flow-accumulation from the DEM. Soil erodibility (K), cover-management (C) and conservation practice (P) factors are prepared by reading, reprojecting, and resampling the soil and LULC layers, then assigning literature-derived K and C values (and P where data exist, otherwise assuming P = 1). For each year of the study period, these five factors are multiplied pixel-by-pixel to generate annual soil-loss maps according to

The resulting decadal series of rasters is analyzed and visualized through erosion-hotspot maps, catchment-mean time-series plots, and per-pixel change-detection. Finally, aggregated results are compared with published literature for validation, and all code, assumptions, and outputs are documented in a Google Colab notebook for full reproducibility.

__________________________________________________________________________________

# 7. Probable Outcomes

- A set of GeoTIFFs with annual soil-loss estimates (2011–2020) at 30 m resolution

- Python notebook and scripts for end-to-end RUSLE processing

- Identification of priority sub-catchments for conservation planning

- Open-source repository for community use and further development

__________________________________________________________________________________

# 8. Work Plan

| Month | Tasks |
| --- | --- |
| June–July 2025 | Literature finalization; data acquisition and preprocessing |
| August–September 2025 | Develop and test factor-calculation modules (R, K, LS, C) in Colab |
| October 2025 | Run full annual soil-loss workflow; perform validation |
| November–December 2025 | Prepare decadal change assessment; write thesis chapters; finalize |

__________________________________________________________________________________

# 9. References

- Renard KG, Foster GR, Weesies GA, McCool DK, Yoder DC. Predicting Soil Erosion by Water: A Guide to Conservation Planning with the Revised Universal Soil Loss Equation (RUSLE). USDA Agriculture Handbook No. 703; 1997.

- Shinde VM, Karale BV, Ingole MD. Hydrological modeling of soil erosion using RUSLE and GIS techniques in a Western Ghats watershed, India. Environ Monit Assess. 2010;165(1–4):111–9.

- Prasannakumar VN, Bantari RR, Vijay K. Soil erosion assessment using RUSLE and remote sensing for a Kerala watershed, India. J Indian Soc Remote Sens. 2012;40(3):539–49.

- Ganasri BAM, Ramesh HA. Assessment of soil erosion by RUSLE model using remote sensing and GIS—a case study of Nethravathi Basin. Environ Earth Sci. 2016;75(16):1417.

- Papaiordanidis Y, Jirka SJ. Implementing RUSLE in Google Earth Engine for rapid soil‐erosion mapping. Comput Geosci. 2019;128:36–45.

- Pandey AK, et al. Sensitivity of RUSLE LS-factor to DEM resolution: A case study in the Himalayan foothills. Geomorphology. 2020;365:107262.

- Chakrabortty T, et al. Hybrid machine‐learning and RUSLE approach for erosion-hotspot identification in east India. Catena. 2021;203:105309.

- Ghosh P, et al. Decadal soil-loss estimation using Python: Integrating ESA WorldCover and IMD rainfall in the Mayurakshi basin. Soil Use Manage. 2023;39(1):112–24.